Problem set 5

- Physical logic gates take a finite time to respond to changes in their input signals. What name is given to this time?

- Hold time.

- Propagation delay time.

c)   Rise time.

d)   Set-up time.

- Express the binary number 1001 in decimal.

a) 9

b) 15

c) 11

d) 13

- Express the decimal number 57 in binary.

a) 111010

b) 111101

c) 111001

d) 110011

- Which of the following is not a form of multivibrator?

a) Astable

b) Monostable

c) Bistable

d) Tristable

- The S-R latch shown here has active high inputs.

a) True

b) False

- A J-K flip-flop has two control inputs. What happens to the Q output on the active edge of the clock if both control inputs are asserted at this time.

a) The Q output remains unchanged.

b) The Q output is reset to 0.

c) The Q output is set to 1.

d) The Q output toggles to the other state.

- A master/slave bistable is formed using two bistable connected in series.

a) True

b) False

- What type of monostable produces waveform of the form shown here?

a) A retriggerable monostable

b) A non-retriggerable monostable

- An astable has two metastable states and produces the function of a digital oscillator.

a) True

b) False

- What is the function of the following circuit?

a) A four-bit shift register.

b) A four-bit memory register

c) A four-bit ripple counter.

- What is the frequency of the output of the following circuit?

a) 1 Hz

b) 16 Hz

c) 4 Hz

d) 8 Hz

- What is the function of the following circuit?

a) A modulo-6 counter.

b) A modulo-12 counter.

c) A modulo-8 counter.

d) A modulo-10 counter.

- In synchronous counters the clock input of each of the bistables are connected together so that each changes state at the same time.

a) True

b) False

- What is the cause of storage time in a bipolar transistor?

a) The 'memory effect of the device.

b) The inertia of the minority charge carriers.

c) The inertia of the majority charge carriers.

d) The time taken to remove excess charge stored in the base region as a result of saturation.

- What is meant by the fan-out of a logic gate?

a) The number of other gates that can be connected to the gate's output.

b) The amount of cooling required by the gate.

c) The physical distance between the output pins on the device.

d) The number of other gates that can be connected to one of the gate's inputs.

- Which of the following statements is incorrect?

a) TTL devices have logic levels of about 3.4 V and 0.2 V.

b) Standard TTL devices have a propagation delay that is dominated by the storage time of the bipolar transistors used.

c) TTL logic has very low power consumption and is therefore widely used in highly integrated components.

d) TTL logic normally operates from a single 5 V supply.

- What should be done with an unused TTL input that is required to be at logical 1?

a) It should be connected directly to the positive supply rail.

b) It should be tied to the positive supply rail through an appropriate resistor.

c) It should be connected directly to the zero volt supply rail.

d) It should be left disconnected.

- In the following PLA, which output implements the logic function ABCD?

a) X			b) Y			c) Z

- The cells in a FPGA may contain registers, look-up tables and memory.

a) True			b) False

- Which of the following statements is incorrect?

a) Some PLDs are programmed using electrically operated switches.

b) Some PLDs are programmed using fuses that are selectively blown.

c) Some PLDs are programmed using mechanical switches.

d) Some PLDs are programmed using anti-fuses that are selectively joined.

- Communications within a microprocessor take place over a number of serial buses.

a) True			b) False

- Which of the following statements is incorrect?

a) Static RAM stores information by energizing or de-energising inductors.

b) Dynamic RAM stores information by charging or discharging capacitors.

c) RAM is volatile.

d) RAM is memory that can be written and read quickly.

- Which of the following statements is incorrect?

a) EPROMs can be erased using an ultraviolet light source.

b) ROM devices must be programmed by the chip manufacturer.

c) ROM devices are non-volatile.

c) EEPROMs can be written to (programmed) as well as read from.

- How many address lines would be found on a 128-kbyte memory device (assuming that this is arranged as an array of 8-bit registers)?

a) 13			b) 19			c) 15			d) 17

- A signal contains components with frequencies up to 10 kHz, although no useful information is contained at frequencies above 6 kHz. What is the minimum frequency at which the signal should be sampled?

a) 6 kHz.			b) 20 kHz.		c) 12 kHz.		d) 14.4 kHz.

- When using anti-aliasing filters it is normal to sample somewhat below the Nyquist rate to allow for the non-ideal characteristics of the filters.

a) True			b) False

- What is the resolution of a 12-bit data converter?

a) 0.00024%		b) 0.41%		c) 0.0041%		d) 0.024%

- What is meant by the “droop” of a sample and hold gate?

a) The voltage by which the input quantity is lowered during sampling.

b) The time taken to take a sample.

c) The rate of decay of the output voltage.

d) The voltage difference between the output voltage and the actual voltage being sampled.

- An analogue multiplexer is a form of electrically controlled switch, based on the use of analogue switches.

a) True			b) False

- What is the primary function of the sample and hold gates in the following arrangement?

a) To allow all the inputs to be sampled simultaneously.

b) To block unwanted input signals.

c) To buffer the input signals.

d) To prevent the ADC from distorting the input signals.

- What is the primary function of the sample and hold gates in the following arrangement?

a) To prevent variations in the load from affecting the output voltage.

b) To buffer the output signals.

c) To hold the outputs constant between updates of the outputs.

d) To allow the outputs to be updated simultaneously.