- Physical logic gates take a finite time to respond to changes in their input signals. What name is given to this time?

a) Hold time.

b) Rise time.

c) Set-up time.

d) Propagation delay time.

Since a gate takes a finite time to respond to an input signal, its output will be delayed relative to that input. Should that input signal and the output of this gate be used as inputs for another logic gate, the signal from the first gate will have a propagation delay time relative to the original signal. Should the delay be substantial, it could result in the new gate being driven by the current, un-delayed signal and the old state of the system from the previous gate that has not yet been able to update its outputs due to its finite response time.

- Express the binary number 1001 in decimal.

a) 9

b) 11

c) 13

d) 15

Since the number 1001 represents 123+022+021+120 = 8+0+0+1 = 910

- Express the decimal number 57 in binary.

a) 111010

b) 111001

c) 110011

d) 111101

Converting from decimal to binary, one can use the method:

572 = 28 + remainder 1

282 = 14 + remainder 0

142 =   7 + remainder 0

72 =   3 + remainder 1

32 =   1 + remainder 1

12 =   0 + remainder 1		taking the remainders from bottom to top gives 1110012

- Which of the following is not a form of multivibrator?

a) Astable

b) Bistable

c) Tristable

d) Monostable

Combinational and sequential logic use only 2 logic levels, 0 and 1. Thus a multivibrator can only switch between these states.  Sometimes the term tristable is used for a logic device that can turn its outputs off (0) but set them to a high impedance so as not to appear to other devices on a bus.  However, this is not a different logic state, but a feature of the “0”.

- The S-R latch shown here has active high inputs.

a) True

b) False

The circles on the inputs show that this device has active low inputs.

- A J-K flip-flop has two control inputs. What happens to the Q output on the active edge of the clock if both control inputs are asserted at this time.

a) The Q output remains unchanged.

b) The Q output is set to 1.

c) The Q output toggles to the other state.

d) The Q output is reset to 0.

“Asserted” means that both inputs are held high, or in the “1” logic state. In this condition, the clock will toggle the output (if it had been1, it will be switched to 0; if it had been 0, it will be switched to 1)

- A master/slave bistable is formed using two bistable connected in series.

a) True

b) False

This is true. The outputs are determined by the first (master) flip-flop while the clock is high, but the outputs of the second (second) flip-flop only change when the clock makes the transition from high to low. The slave is disabled before the new signals arrive at the master, so the output of the slave is steady until the clock makes the transition from high to low. In this way, the “race” condition, where a downstream device may be enabled while the output of the previous gate is in transition.

- What type of monostable produces waveform of the form shown here?

a) A retriggerable monostable

b) A non-retriggerable monostable

Here we can see that the device is normally at 0, but when it is triggered, it changes its state from 0 to 1 for a set time period, , before returning to its default state, 0.  This is a monostable device. On the last clock pulse we can observe that the device can be re-triggered. That is, if it is triggered before it has reset to 0, it will remain high from the second trigger for time period, . If the device had ignored the second trigger and returned to 0 a time  after the first trigger, it would be a non-retriggerable monostable.

- An astable has two metastable states and produces the function of a digital oscillator.

a) True

b) False

An astable device combines two devices so that the first one sends the second one into a metastable state, but that metastable state switches the first one into its metastable state…and so on and so on. Thus, the output of a device will oscillate between 0 and 1.

- What is the function of the following circuit?

a) A four-bit shift register.

b) A four-bit memory register

c) A four-bit ripple counter.

This is a series of flip-flops in toggle mode. Whenever the toggle goes from 1 to 0 (10), the output will change to the opposite state it had. So, initially at 0, when the first clock pulse goes from high to low (10), the output of Q0 will switch from 0 to 1.  On the second clock pulse falling edge (10), the output Q0 will also toggle from 10, but this will ripple on, triggering the second flip-flop Q1 output to go from 0 to 1. On the third clock pulse falling edge (10), Q0 will go from 0 to 1, but Q1 will remain at 1 since its clock (which is Q0) has not made a transition from 10 (it made the transition 01 which will not act as trigger).

Looking at Q0 and Q1, on clock pulse 0 they read Q1=0, Q0=0; on clock pulse 1 they read Q1=0, Q0=1, and on clock pulse 2 they read Q1=1, Q0=0.  On clock pulse 3, they would read Q1=1, Q0=1.  That is, the pair gives the binary word that represents the number of clock pulses. We can assume Q2 and Q3 will follow this, so it is a 4-bit ripple counter.

- What is the frequency of the output of the following circuit?

a) 1 Hz

b) 4 Hz

c) 8 Hz

d) 16 Hz

A 14 stage binary divider will divide by 214=16384. This means the output will be 131072 Hz/16384=8 Hz

- What is the function of the following circuit?

a) A modulo-6 counter.

b) A modulo-8 counter.

c) A modulo-10 counter.

d) A modulo-12 counter.

A three-stage counter without any reset circuitry will count from zero to seven and then repeat (giving a modulo-8 counter). The presence of the reset circuitry in this arrangement detects the count of 6 and then resets the circuit to zero. Thus the circuit counts from zero to five and then repeats, and is thus a modulo-six counter.

- In synchronous counters the clock input of each of the bistables are connected together so that each changes state at the same time.

a) True

b) False

To avoid the delay in the ripple counter, where the clock transition 10 has a propagation delay to the output Q0, which then ripples into the inputs that, after a propagation delay, determine Q1, one could connect all the toggle inputs to the same clock. We would then need additional circuitry to detect which inputs should change and which should remain the same, and adjust the J inputs of the different flip-flops accordingly. Thus, only the first, LSB flip-flop is operated in toggle mode, the others are in J-K flip-flop mode.