Clearly a sinusoid with an amplitude of 8. It crosses zero at -10, 30 and 70 ms, making it a period of =80 ms. The time factor in the sinusoid is t =2t/, or in this case (78.5 radians/s)t. Now, when t=0, the sine has already risen from 0 and is positive, indicating that the phase factor must be positive. While it might be difficult to judge 0.7078 amplitude units, the only choices are  /4, and at t=0, a -/4 would yield a negative number. Thus, the correct anwer is 8∙sin78.5∙t+π4.

Capactive reactance is defined as Xc=1ω∙C , where  (rad/s) = 2f (Hz). Here we have f=10 Hz, giving =62.8 rad/s, and with a capacitance of C= 5 F, Xc = 1/(62.85) = 0.003

The resonant frequency of an R-L-C series circuit is ω=1L∙C . So, for L=5 H, and C=1x10-6 F, we have that =447 radians/s. Unfortunately the units of the answer are in Hz. Thus, since f (Hz) = /(2), the frequncy f = 71.2 Hz

Tracing through the top terminal through the chain, we see can draw the circuit like this:

Now, R11 and R12 ae in series with each other, and the series pair is in parallel with R6:

Req1=(R11+R12)//R6.

This Req1 is in series with R5, and this series pair is in parallel with R4:

Req2=(Req1+R5)//R4

This Req2 is in parallel with R3:

Req3=Req2//R3

This is in series with R2, and the series pair of Req3 and R2 are in parallel with R1:

Req = (R2+Req3)//R1

Then you just need to remember that series of a and b is a+b, and parallel of a and b is a*b/(a+b).

Req1 = (5+12)*2/((5+12+2) = 1.79 k

Req2 = (1.79+4)*2/((1.79+4)+2) = 1.49 k

Req3 = 1.49*10/(1.49+10) = 1.29 k

And

Req = (1.29+4)*1/((1.29+4)+1) = 3.19 k

This one is deceptively easy. Here we have a situation where the voltage across all the resistors is the same, 12V. The total current is given as It=21 mA, and the current flowing through the parallel pair 3//4 is I34 = 3 mA. That means the current flowing through the parallel pair 1//2, I12=It-I34=18 mA.

Now, the resistance 1//2 should be (1*2)/(1+2) k = 666.7 . That should be the same as 
V/I12 = 12V/18mA=666.7 . Since it is, that leg is OK and the problem is in the 3//4 leg.

The parallel resistance 3//4 should be 3*4/(3+4) k = 1714.3 k. But the current measured flowing though this pair would indicate that the resistance is R34 = 12 V/3 mA = 4 k, the value of R4. That means that R3 is no longer in the circuit and is open.

By convention, we assign the voltage so that the arrow points to the high potential end and we assign it a positive value. If we are not sure, one can assign an arbitrary direction, but is the solution comes out negative, or if it is pointing to the low potential, it will have a negative value. This tells you that the high potential end is actually at the tail of the arrow as we have drawn it!

So, here we have a voltage source of E= 20 V. That means its potential at point A is 20 V higher than the potential at point D. Now for convenience we will take point D, which is the reference point for all the potentials shown, to be at 0 V (remember that potential needs a reference point). Now, potential at point A is VA=20V above the potential at point D, where VD=0V, and the arrow E points to the higher potential and is assigned a postive value.

From point A to B, the potential drops V1=4 volts across the resistor. Thus, the point B is at a lower potential VB= 20-6= 16 V.  Note that the voltage arrow V2 is pointing to the higher potenial end and V2 is assigned a positive value.  From point D to C, we see that the potential difference across the resistor is V3=6V.  Now point D is at 0V, and we know that point B is at 16V, so point C must be at higher potential than point D by 6V. Thus, point C is at Vc=6V, and again V3 is pointing to the high potential end and is given a positive sign.

That means that there is a potential difference from VB=16V to Vc=6V, or a 10V potential drop with VB>VC. Unfortunately, our choice of direction for V2 is pointing toward the low potential end. Thus, we have that V2=-10V, to warn it is pointing to the low potential end.

The valence band is at a lower potential for electrons

The Apparent power is S= VrmsIrms VA,  and the power factor is the cosine of the phase angle, , between voltage and current. The Active power is then the apparent power times the cosine factor, or P=Scos() W. Here the Apparent power is 804 = 320 VA, and the power factor is cos(20) = 0.94. This gives an Active power of 300.7 W

The point of a transistor is to take electrons from the emitter and pass them to the collector, completing the circuit, but to be able to stop that flow with a low current/voltage to the base that sets up a potential barrier. When the transistor is “on”, you do want those negative electrons to pass through the base to the collector without recombining with positive holes in the base. Thus, you make the base thin in order to allow those electrons to flow without recombining.

An ideal operational amplifier would have infinite input impedance, Ri, so as not to load the input circutry. It would have an infinite openloop gain, A, to make it controlable by the external feedback components. Finally, it would have no output impedance, Ro, so that any voltage created by the op-amp would be tranferred to the external circuitry without dividing it.

Here we have to remember that the voltage across a set of resistors in parallel is the same. Thus, the 12 V tells you that none of the resistors are shorted otherwise it would be 0 V (the voltmeter would be connected to ground on both sides). However one could still be open.  The equivalent resistance of the parallel network would be the voltage, 12V, divided by the current, 10 mA = 1.2 k.  When we calculate the equivalent resistance of the network, we find that it is 1.2 k, so there is nothing wrong with the circuit.

Since electrons are the more mobile charge carriers, the potential energy diagrams are set up with the potential energy of electrons increasing in the upward direction (in the direction of C). Once the junction is formed, the free electrons in the n-type material near the junction diffuse across the junction and re-combine with the excess positive holes in the p-type material. This leaves a net positive charge on the n-side of the junction (lower potential energy for electrons) and a net negative charge on the p-side (higher potential energy for electrons). Thus, electrons are given energy to promote them from the lower electron energy valence band, F, into the higher electron potential energy conduction band A&B. However, the electrons in the conduction band will drift to a lower electron potential energy across the junction (towards the positive side of the junction), B.  This positive side of the junction is, as we said above, the n-type material

On an FET, we have a gate that controls the flow of electrons, a source of electrons, and a drain of electrons. However, in an FET the direction of flow can be source to drain or drain to source depending upon the bias. Fulfilling a similar function on a BJT transistor, we have a base, emitter and collector, where the electrons do flow from the emitter to collector. That terminology, in turn, developed from vacuum tubes (valves) that had a grid, cathode and an anode, where electrons left the cathode toward the anode, but the control of the flow was done with the bias in the grid.  Remember, the current flow is in the opposite direction of the electron flow!

The point of Gray code is that only one bit changes as the number is increased. It is formed by first counting to 1 in binary

0

1

Then mirroring these numbers

0

1

1

0

Then add a 0 to the initial numbers (0 and 1) and a 1 to the mirrored numbers (1 and 0:

00

01

11

10

Then repeat

00			000		0

01			001		1

11			011		2

10			010		3

10			110		4

11			111		5

01			101		6

00			100		7

and again

000			0000		0

001			0001		1

011			0011		2

010			0010		3

110			0110		4

111			0111		5

101			0101		6

100			0100		7

100			1100		8

101			1101		9

111			1111		10

110			1110		11

101			1101		12

011			1011		13

001			1001		14

000			1000		15

When A is high, the inputs are 1 0 which results in a 1 for an exclusive OR gate, but this is an exclusive NOR which negates this to a F=0. So, A=1, F=0.   When A is low, the inputs are 0,0, and the output of an exclusive OR would be 0. But the Exclusive NOR negates this so that F=1. Now we have A=0, F=1, or F is the opposite (negation) of A

Short out the terminals (conect a wire across them), and the current will  avoid R3 and flow through the wire. Thus the current will be 10 V/ 10k = 5 mA.

The diffusion current is the current of the free charges in the doped material neutralizing each other across the junction to create the depletion region and set up the electric field. The higher the doping level the more free charges in the material and the higher the diffusion current.

Negative feedback will decrease the gain from the open-loop gain. However since the gain-bandwidth product is constant, this will increase the bandwidth. Op-amps using negative feedback derived from the output voltage will tend toward an ideal op-amp, with input impedances and output impedances tending toward an ideal amplifier by factors of 1+AB, the product of the open-loop gain, A, times the closed loop gain, B.

Operating a transistor in a digital device one wants to have the transistor either fully on or fully off. Thus, we operate a BJT transistor in the saturation or cut-off region.

An ideal pn junction diode is operating in the forward bias region. The change in diode voltage, that

will cause a factor of 9 increase in current, is

55 mV

35 mV

45 mV

85 mV

Id=Is*(exp(40*Vd)-1)

With Vd=0.1, exp(40*0.1)>>1 so IdIsexp(40*Vd)

Id1/Id2 = 9exp(40(Vd1-Vd2))

Ln(Id1/Id2) = ln(9)  40(Vd1-Vd2)

2.19/40 V  Vd1-Vd2

55 mV Vd1-Vd2

In the voltage regulator circuit in the figure, what is the maximum load current iL that can be drawn?

1.4 mA

1.8 mA

2.3 mA

2.5 mA

Well, the voltage across the diode is Vd, but to get maximum current across the load, the voltage across the diode, Vd, should be just at the the Zener voltage, Vz, and the diode would be just be at the point of passing current.  All the current would go into the load, and increasing the voltage any further would just pass the extra current through the Zener. The voltage across the diode just as it turns on, Vd, is the supply voltage, Vs, divided across the R1=15 k and RL resistors, or Vd=Vs*RL/(R1+RL). Solving for RL gives us RL=Vd*R1/(Vs-Vd).  The current across the load resistor is just the voltage across the load, Vd, divided by RL. Or, Imax=Vd/RL.  Subtituting in for RL, and using the fact that Vd=Vz, we get

Imax=(Vs-Vd)/R1 = (Vs-Vz)/R1=(30V-9V)/15000  = 1.4 mA

14

If we short out the power supply, we can look at the equivalent resistance from A to B. This will be the Thevenin resistance. If we start at B, we find that R1, R3 and R4 are all connected together at B. The upper end of R1 and R3 are connected together, putting them in parallel. This parallel junciton is connected to R2, and thus the parallel pair R1//R3 is in series with R2. The upper end of R2 is connected to the upper end of R4 at terminal A, and so R4 is in parallel with R1//R3+R2.

We can break it down and calculate the parallel pair R1 and R3 as 
Req1=R1*R3/(R1+R3), or substituting in values, we get Req1=666.7

This is in series with R2, giving a Req2=Req1+R2 = 4.66 k. Finally, this Req2 is in parallel with R4, giving the total, Thevenin resistance Rth = Req2*R4/(Req2+R4). Again substituting in values gives Rth = 2.41 k

To get the voltage across A and B, the Thevenin voltage, we have to put the power supply back in the circuit. Now we need to calculate the total current, which means we need to look at the equivalent resistance seen by the power supply. From the figure, we can see that this is R2 and R4 in series, and this series resistance in parallel with R3. Lastly, R1 is in series with the parallel pair. So we have R1+R3//(R2+R4). The series of R2 and R4 is 9 k, and this is in parallel with R3=1 k. So this would be 9*1/(9 + 1) k, or 0.9 k. Finally we add the series resistance of R1 = 2 k, and we get an equivalent resistance as seen by the power supply of Rtot=2.9 .

Finally, the current is I = V/Rtot =  10V/2.9 k = 3.45 mA. The voltage after R1 is just 10 V – I*R1, or 10V-6.89V = 3.1V.  Now, this voltage, Vp, is divided across the two resistors R2 and R4, and the voltage across R4 is Vp*R4/(R4+R2). This is just the voltage divider equation. So V4 = Vth = 3.1*5/(5+4), or Vth = 1.72 V