TFY4185 2020H Sampling 1

- A signal 𝑥(𝑡)=100  cos(24000𝜋 𝑡 )is first sampled with a sampling period of 50 μsec and then passed through an ideal low pass filter with cutoff frequency of 15 KHz. Which of the following frequency is/are present at the filter output?

(a) 12 KHz and 8 KHz

(b) 8 KHz only

(c) 12 KHz and 9 KHz

(d) 12 KHz only

Soln. Signal frequency, fm = 24x103 / (2) = 12 kHz, giving a Nyquist frequency of 24 kHz

Sampling period = Ts = 50x10-6 sec

So the sampling frequency = /Ts = 1/(50x10-6 sec) = 20 kHz, which is below the Nyquist

After sampling, signal will have frequencies fm and fs-fm

So, we will have a 12 KHz and 20 KHz – 12 KHz = 8 KHz

After filtering, the frequencies present will be 12 KHz and 8 KHz

Option (a)

TFY4185 2020H Circuit Analysis 5

- Assuming ideal 30 V voltage source and an ideal 3A current source in the circuit below, what is v1?

- 60 V

- 15 V

- 45 V

- 30 V

- 105 V

60 V

Superposition: ideal voltage source has no internal resistance; ideal current source has infinite resistance:

To find voltage at v1 due to the 30V supply, replace the current source by an open circuit (infinite resistance). This is now a 30 V source across the series resistance of (10 + 30 + 20)  = 60 . Ohms law says the current from the voltage source is Iv = 30 V/60  = 0.5 A. The voltage drop across the 30  resistor due to the voltage source is v1v = 0.5 A  30  = 15 V.

To find voltage at v1 due to the 3 A supply, replace the voltage source by an short circuit (= wire = zero resistance). This is now a 3A source across a parallel pair of 30  // (10 + 20)  = 30  // 30  = 3030 2 / (30 + 30) = 15 . Ohms law says the voltage across each leg of this parallel pair from the current source is v1i = 3 A  15  = 45 V. Thus the voltage drop across the 30  resistor due to the voltage + current source is v1 = 15V + 45 V = 60 V.

TFY4185 2020K Circuit Analysis 3

- In the circuit below, what is the value of R1?

(A) 100 Ohms		(B) 50 Ohms		(C) 25 Ohms		(D) 2500 Ohms

The voltage drop across the 60 Ohm resistor is 30 V (100 V at Power supply end– 70 V). The current thought the 60 Ohm resistor is I = V/R which gives 30 V / 60 Ohms = 0.5 A = I

The voltage across R1 is 50 V (70 V at junction with the 60 Ohm resistor – 20 V across R2). The same current though the 60 Ohm resistor flows though R1 (and R2), so V1/I = R1 gives:

50 V / 0.5 A = 1000 Ohms.

TFY4185 2020H Capacitors 6

- Determine the effective capacitance of the following arrangement:

- 45.0 micro-Farads

- 19.6 micro-Farads

- 11.1 micro-Farads

- 115.0  micro-Farads

We have a 30 F in series with a 60 F capacitor that gives an equivalent capacitance of 30*60/(30+60) = 20F. This is in series with a 25 F capacitor, with a equivalent total capacitance of 20+25=45 F

TFY4185 2020K Thevenin 1

- Calculate Rth and vth for the following circuit:

(A) 1 V, 5/6 Ohms	(B) -2 V, 6/5 Ohms	(C) 2 V, 5/6 Ohms	(C) -1 V, 6/5 Ohms

Take R1 = 1, R2 = 2 and R3 = 3. To calculate Rth, replace the ideal current source with an open circuit and calculate the equivalent resistance between the white terminals on the right of the diagram.  This is

Rth = R1//(R2+R3) = 1(2+3)/(1+(2+3))  = 5/6 .

To calculate Vth, replace the current source and calculate the voltage drop across it creates across R1.

To do this, first calculate the voltage drop across the equivalent resistance from the 2 A supply.  Req = R3//(R2+R1) = 33/(3+3) = 9/6 . The 2 A across this Req will give Veq = 2A 9/6  = 3 V.  But, this 3 V is divided across R2 and R1 in the leg of the circuit over which Vth is measured.

Thus, using the voltage divider equation, V1 = Vth = VR1/(R1+R2) = 3/3 = 1V.

Thus, a voltage source with a 5/6  resistor in series will look the same to any circuit connected to the white terminals on the right.

TFY4185 2020H Transient Response 1

- In the figure below,  2.5 A ideal current source is run with the switch closed for a very long time. What is the magnitude of the voltage Vx immediately after opening the switch at t = 0?

- 50 V

- 0 V

- 25 V

- 13.3 V

We have a 20  resistor in parallel with two, 5 H inductors that are in series. That make it a 20  resistor in parallel with an equivalent 10 H inductor. When the switch is closed, the current source connects across this parallel pair, which itself is in parallel with another 40  resistor.

The “very long time” means that the circuit has reached steady state, and the current is constant through the energized inductors at IL = 2.5 A.  In this steady state, DC condition, the inductors represent a wire that shorts out all the other parallel resistance paths. That is, the current source connects across a wire (since it is an ideal current source, it can do this).

When the switch is opened at t=0, we create two separate circuits. The first is a closed loop of the 2.5 A current source across the 40  resistor. In the second circuit, which does not exchange current with the first, the current through the inductors does not want to change, and the energized inductors initially provide 2.5 A of current to this second, closed loop. This second circuit is the one that flows current through the resistor of interest, and the magnitude of the voltage across this 20  resistor is Vx = 2.5 A  20  = 50 V

The question might come up as to whether this second loop interacts with the first. Well, remember potential is measured relative to a reference, and current flows from high to low potntial. So a loop like this:

the current will flow the current supply, through the resistor ad back to the current supply. It is just that we reference the bottom of the current supply at ground potential or 0V

If we add a power supply, we lift the 0V reference point above ground:

The original loop still sees a high potential to low potential across the power supply, and so the current sill flows through this loop. All points below the resistor and the current supply are at the same potential, and cannot see the lower ground potential through the voltage supply.  Thus, the current does not flow through the voltage supply to the 0 V ground potential.

Think what would happen if one side of the power supply could see the lower potential through the power supply itself. There would be a direct short to the lower potential through the power supply, and there would be no potential difference across the terminals of the power supply. If we attached a resistor across the terminals, it would see both sides at the same potential, and we would never get any current to flow through any attached circuit.  That is not much of a power supply!

TFY4185 2020H AC Circuits 2

- In figure, A1, A2, and A3 are ideal ammeters that read rms current. If A1 reads 5A, A2 reads 12A, what will A3 read?

- 13 A

- 17 A

- 12 A

- 7 A

- 5 A

Since it is a sinusoidal current with resistive and reactive components, there will be phase shifts between the applied voltage and current. Since R and C are in parallel, the voltage across them will be the same, and we are interested in the currents across these elements.

The current across the resistor is in phase with the applied voltage, and the current across the capacitor leads the applied voltage by 90 degrees, so the phasor for the current in the capacitor is upward relative to the voltage. As a result, the current across the resistor and capacitor parallel pair is:

Irms= IRrms2+ICrms2=52+122A=13 A

TFY4185 2020K RL Circuits 1

- In the circuit below, the frequency is adjusted until the magnitude of the gain, |(vo /vs)|, is equal to 0.6. What is the value of the frequency?

(A) 20 radians/s		(B) 20 Hz		(C) 40 radians/s		(D) 40 Hz

The impedance divider will cause the voltage Vo = Vs*ZR/ZT, where ZT is the total, complex impedance of the inductor and resistor, ZT = R + jL. This gives a transfer function H() = Vo/Vs = R/(R + jL), and a gain |Vo/Vs| = R/(R2 + 2L2)½. If this gain, g=0.6, we have:

= R/L(1 - 1/g2)½ , or substituting in values,  = 20 R/s

TFY4185 2020H RLC Circuits 1

- A series LCR circuit consisting of 𝑅 = 10 Ω, |𝑋𝐿 | = 20 Ω 𝑎𝑛𝑑 |𝑋𝐶 | = 20 Ω is connected across an a.c. supply of 200 V rms. What is the rms voltage across the capacitor?

- 400 V  -90o

- 400 V  +90o

- 200 V  -90o

- 200 V  +90o

For a series RLC circuit, the current is the same through all the elements, and we measure voltages relative to this applied current. The total impedance of a series RLC circuit is Z = R + jXL-JXC. In this case, XL = XC, so that Z = R. Thus the applied voltage and current are in phase, and the current is Irms = V/R = 200 Vrms/10  = 20 A.

The voltage across the capacitor is Irms(-jXC) = 20(-j20) V = -j400 V

That is a voltage that lags the applied current by 90 degrees, giving a -90 phase angle.

TFY4185 2020H AC Power 1

- An AC source of RMS voltage 20 V with internal impedance 𝑍S = (1 + 2𝑗)Ω feeds a load of impedance 𝑍𝐿 = (7 + 4𝑗)Ω in the figure below. What is the reactive power consumed by the load? (just the load, not the source resistance)

16 VAR

- 16 VAR

- 8 VAR

- 32 VAR

- 28 VAR

The reactive power, Q = Vrms*Irms*sin() given in Volts-Amperes-Reactive, or VAR. We can look at phasor notation, where each leg of the triangle is a magnitude

Now, we can multiply by the magnitude of the current (which is the rms current) to obtain rms voltage V:

If I multiply by the magnitude of the rms current again, I obtain the power dissipated:

Which relate to Active power, P; Reactive power, Q; and Apparent power, S:

The book then gives the components of VI using the sine and cosine of the phase angle, .

S = VI (=I2Z), Q = VIsin() (=I2X), and P = VIcos() (=I2R)

Now, here we are given V and the can calculate the equivalent impedance of the two series elements as:

Z = ZS + ZL = (1+2j) + (7 + 4j) = 8 + 6j ,

This gives a current: I = V/R = 20 V/(8 + 6j )

The magnitude of I2 is I times its complex conjugate, or 202/[(8 + 6j) (8 - 6j)] = 400/(64+36) = 4.

To get Q we multiply I2 times the reactance of the load, which is the imaginary part of the load impedance, 4, to get the reactive power, Q= 16 VAR

TFY4185 2020H Op Amps 10

- For the Operational Amplifier circuit show below, what is Vo? You may assume an ideal operational amplifier.

- -0.5 V

- +0.5 V

- -1 V

- -2 V

- +1 V

- +2 V

On the non-inverting input, the 1V divided across the two 1k resistors will keep the non-inverting input at 0.5 V.

To balance this, the inverting input must also equal 0.5 V (the rule is that V across the inputs is zero).

Now, the current from the source toward the inverting input, V_, if V_= 0.5 V (to keep V=0), is:

I1 = (1V-0.5V)/1k = 0.5 mA

The current from the output of the op-amp to the V_ input, where again V_=0.5V, is:

I2 = (Vo-0.5V)/2 k = Vo/2 mA – 0.25 mA

Now, since no current can flow into the op-amp, Kirchhoff’s current law says I1+I2 = 0, or I1 = -I2

Substituting in for the currents, we have:

0.5 mA = –Vo/2 mA + 0.25 mA, or Vo = –0.5 V

One can also treat the inverting input feedback as a voltage divider where the voltage drop of 1-Vo volts drops across a 1 k and 2 k series resistance, and one is taking V_=0.5V between the resistors. The voltage after the 1 k resistor is:

V_= ½ V = (1-Vo) 1 k/(1 k+2 k)

Or ½ V = 1/3 V – Vo/3

Which also gives Vo = – 0.5 V

TFY4185 2020H Op Amps 11

- The gain of an operational amplifier falls with frequency as -6 dB/octave. If it has a cutoff frequency at 20 Hz when the mid-band open-loop voltage gain is 1,000,000  what is the unity-gain cutoff frequency?

- 20 MHz

- 2 MHz

- 1 MHz

- 20 Hz

A gain change with frequency of -6dB/octave represents a first order system, for which the gain-bandwidth product is constant. Thus, the gain-bandwidth product= 20 Hz*106 = 20 MHz-gain is constant and equal to the Unity-gain bandwidth.

TFY4185 2020H BJT 8

- The transistor amplifier below has a saturation voltage VCEsat = 0.2 V and a current gain of 50. What is the minimum base current (IB) required to drive the transistor in the figure to saturation?

- 56 micro-amps

- 140 micro-amps

- 60 micro-amps

- 3 micro-amps

The base current will cause a collector current, IC, to flow through the transistor. The collector current will be related to the base current by: IC=hfeIB, where hfe = 50 is the current gain. The voltage between the collector and emitter is VCE = 3 V – IC1 k. At saturation, VCE =0.2 V. Thus:

0.2 V = 3 V – IC1 k, or IC = 2.8 mA.

The base current that will achieve a collector current of 2.8 mA = 50IB, or IB = 56 A

TFY4185 2020H FET 8

- If properly biased JFET acts as a:

- Voltage controlled current source

- Current controlled current source

- Voltage controlled voltage source

- Current controlled voltage source

The JFET uses a voltage to change the width of the depletion region in the drain-source channel, controlling the current that can flow. There is a non-linear relation between the gate voltage and the current. There is only a small leakage current from the gate due to the p-n junction bias.

TFY4185 2020H FET 9

- Input impedance of MOSFET is

- More than that of a BJT and JFET

- Less than that of a JFET but more than a BJT

- More than that of a JFET but less than a BJT

- Less than that of a JFET and BJT

Since the MOSFET does not use a pn junction, as both BJT and JFET transistors use, it is not subject to the leakage (drift) currents. Instead, the MOSFET input is a capacitor that uses an electric field to change the width of the channel. Since the leakage currents on a capacitor are exceptionally small, it has a higher impedance than either a BJT of JFET.

TFY4185 2020H Semiconductors and Diodes 9

- In an intrinsic semiconductor, the Fermi level, to first approximation,

- Lies at the center of the energy gap.

- Is near the conduction band.

- Is near the valence band.

- May be anywhere in the energy gap.

The Fermi level is the point in the semiconductor where there is a 50% chance of finding an electron. To a first approximation, all the electrons are in the valence band, and none are in the conduction band. Thus, there is a 100% chance of finding electrons in the valence band and 0% of finding them in the conduction band, so that midway between there is a 50% chance.

TFY4185 2020H PN Junctions 1

- The leakage current of a PN junction is caused by:

- Heat energy

- Chemical energy

- Barrier potential

- Majority carriers

The leakage current is the result of random thermal events where the heat energy can cause valence electrons to move to the conduction band, creating electron-hole pairs within the depletion region. These are drawn to the N and P materials, respectively, creating a drift or leakage current. This current causes a slight decrease in the barrier potential and internal electric field, which increases diffusion to rebuild the field. Thus, for an unbiased junction, the drift and diffusion currents are equal and opposite.

TFY4185 2020H PN Junctions 2

- The physical width of the depletion region within a PN junction is reduced when the junction is under:

- Forward bias

- Zero bias

- Reverse bias

- All of these

The width of the depletion region reduces as the forward bias provides excess holes to the P side of the junction and electrons to the N side. These extra positive charges in the P side provided by the bias push the existing positive charges in the material into the depletion region and closer to the N side. Similarly, the extra negative charges provided by the bias to the N side push the electrons in the material into the depletion region and closer to the P side. This narrows the depletion region width and enables the electrons and positive charges to recombine, allowing the current to flow through the device.

TFY4185 2020H Semiconductors and Diodes 10

- A Zener diode in the circuit shown in the figure below has current of 5mA as it begins to break down (a knee current), and a maximum allowed power dissipation of 300mW. What are the minimum and maximum load currents that can be drawn safely from the circuit below, keeping the output voltage V0 constant at 6V?

- 10mA, 55mA

- 60mA, 180mA

- 5mA, 110mA

- 0mA, 180mA

Keeping the output voltage at 6 V means that this is the Zener voltage, VZ. That is, when there is just 6 V across the diode, it is passing 5 mA. If the voltage tries to go above that level, the diode conducts more (passes more current) to keep the voltage across it at 6V.  Now, the diode can only dissipate 300 mW of power. Thus, when there is 6 V across it, the power dissipated in the diode is is IZVZ. The maximum current will then be when 300mW = IZmax6V. Thus, the maximum current the diode can pass is IZmax=300/6 mA = 50 mA.

How much current will the diode normally pass. Well we have a 9V supply, which maintains 6V across the diode. The current from the supply across the 50  resistor must go through both the diode and the load. So the current supplied by the power supply drops from 9V to 6 V across the resistor, and therefore I = (9-6)V/50 = 60 mA.

We cannot take more than IZmax= 50 mA through the diode, so the load must take at least I - IZmax=(60-50) = 10 mA.

At the other end, when the diode first turns on, the current through the diode is 5 mA, with the rest of the 60 mA going through the load. Thus, the load must take the difference between I-IZon = (60-5) = 55 mA as a maxmum current.

TFY4185 2020H Semiconductors and Diodes 11

- In the circuit below, D1 and D2 are identical diodes. Choose the expression that relates the voltage across the diodes, V, to the total current, I, in the circuit (you may assume =1, and q is the electron charge, e).

- V = (kT/q)log(I/Io)

- V = (kT/q)sinh(I/2)

- V = (kT/q)sinh-1(I/2)

- V = (kT/q)(e-1 -1)

For =1, the text shows the current to voltage relationship in forward bias as (Eq 17.1 in text) I = Io(eeV/(kT)-1). In this case, one diode is forward biased and the other is reversed biased, which means the reverse biased diode is passing Io current. Thus the total current, I = Io(eeV/(kT)-1) + Io = IoeeV/(kT). We just need to invert this to get the expression with V in terms of I. This is:

V = (kT/e)log(I/Io)

TFY4185 2020H Combinational Logic 12

- For the circuit shown below the output F is given by:

- F=0

- F=1

- F=X

- F=X̅

The truth table for exclusive or gates is:

A	B|	Y

0	0|	0

0	1|	1

1	0|	1

1	1|	0

Thus, the output of the first gate for the input X (XOR) X is 0

The second gate has input 0 (XOR) X. If X is 1 the output is 1, and if X = 0, the output is 0. Thus, the output of this gate is X

The third gate is X (XOR) X, which, as in the first gate, is 0. Thus, the output F = 0.

TFY4185 2020H Combinational Logic 13

- Which of the following Boolean expressions correctly represents the relation between P, Q, R and M1 in the logic circuit below?

- M1 = (P XOR Q) XOR R

- M1 = (P OR Q) XOR R

- M1 = (P AND Q) XOR R

- M1 = (P NOR Q) XOR R

The logic gates give the expression M1 = [P͞Q(P + Q)] XOR R. Using De Morgan’s Law, this can be written as

M1 = [(P̅ + Q̅)(P + Q)] XOR R. If we look at the expression in [], we find it is an expression for an XOR. To confirm we can to the truth tables shown below. Thus, the expression becomes M1 = (P XOR Q) XOR R

| P + Q¦ Y 0 0¦ 0 0 1¦ 1 1 0¦ 1 1 1¦ 1 | P̅ + Q̅¦ Z 1 1¦ 1 1 0¦ 1 0 1¦ 1 0 0¦ 0 | Y Z¦ [Result] 0 1¦ 0 1 1¦ 1 1 1¦ 1 1 0¦ 0 |
| --- | --- | --- |

TFY4185 2020H Combinational Logic 14

- What is logic function implemented by the circuit below (ground implies a logic “0”)?

- F = P (XOR) Q

- F = P (XNOR) Q

- F = P (AND) Q

- F = P (OR) Q

Redrawing the circuit with the inputs set to 0 or 1 yeilds:

Which lets us do a truth table for P and Q

P	Q|	F

0	0|	0

0	1|	1

1	0|	1

1	1|	0

Which we can recognize as a truth table for an XOR gate.

TFY4185 2020H Sequential Logic 7

- What is the modulus of the counter shown below

- 6

- 7

- 8

- 9

- 5

This is a modulo 6 counter. NAND output connected to asynchronous clear inputs of each flip-flop. As long as NAND output is high if will have no effect on counter. When low it will clear all flip-flops.

The input to NAND are outputs of B and C flip-flops, so, NAND output low when

𝑄𝐵 = 𝑄𝐶 = 1 i.e. it occurs when the flip-flops go to 101 and then to 110 that generates a reset. It will count as follows:  000 → 001 → 010 → 011 → 100 → 101 → 110 (temporary state to clear the FF) so six states only, hence mod 6.

TFY4185 2020H Data Conversion 3

- An n bit ADC that can accept a maximum output represents a voltage of V has a voltage resolution of:

- V/(2n-1)

- V/2n

- V/n

- 2V/n

If we take an n=3 binary counter as an example, we have 2n = 8 digital output states. However, we start from 0 and go to V in steps increasing the voltage by 1 resolution element each time. Thus, there are 2n-1 increments

| Binary out | Decimal out | Voltage in (down from V) | Voltage in (up from 0) |
| --- | --- | --- | --- |
| 000 | 0 | V-7*res = 0V | 0+0*res = 0V |
| 001 | 1 | V-6*res | 0+1*res |
| 010 | 2 | V-5*res | 0+2*res |
| 011 | 3 | V-4*res | 0+3*res |
| 100 | 4 | V-3*res | 0+4*res |
| 101 | 5 | V-2*res | 0+5*res |
| 110 | 6 | V-1*res | 0+6*res |
| 111 | 7 | V -0*res = V | 0+7*res = V |

So, 7*res=V(2n-1)*res = V means that res = V/(2n-1).